| Date | 1 July 2009 OD49 |  |  |  |
| --- | --- | --- | --- | --- |
| Time (UT) | Activity |  |  | Signature |
|  | Planned DTCP: 49; 2009-07-01T16:30:00Z; 2009-07-01T21:30:30Z; NNO. | SDS/TWG/DKG |
|  | The SCOS stack on the MCS for execution of today’s tests was LL_SC_PID_.ws042 |  |  | SDS/TWG/DKG |
| 16:30 | Acquisition of Signal TM looks fine PUMPHTRTEMP 1.85K PUMPHSTEMP 14.39K EVAPHSTEMP 2.65K SHUNTTEMP 1.64K EMCFILTEMP 4.09K SL0TEMP 1.64K PL0TEMP 1.65K OPTTEMP 4.12K BAFTEMP 4.12K BSMIFTEMP 4.09K SCAL2TEMP 4.25K SCAL4TEMP 4.16K SCALTEMP 4.07K SMECIFTEMP 4.05K SMECTEMP 4.06K BSMTEMP 4.06K SUBKTEMP 286.4 mK – CREC was recycled in OD47 (Started at 11:02UT on 30th June 2009) L0 C100_1_T225= 1.65K L0 C100_3_T226= 1.67K L0 C100_2_T227= 1.64K L1 C100_3_T235= 3.32K L1 C100_3_T236= 3.86K L2 C100_3_T250= 12.32K L2 C100_4_T252= 12.27K L3 C100_4_T246= 12.34K L3 C100_4_T247= 12.39K Telescope M1 T331->T337 = 86.2K |  |  | SDS/TWG/DKG |
| 17:10 | Start of transition to PHOT_STBY mode, before mass memory (VC2) dump finished. BSM_ON / H_COP_SPI_MBSN OBSID: 0x00002096 BSM_ON / H_COP_SPI_MBSI OBSID: 0x00002097 BSM initialisation was normal. |  |  | SDS/TWG/DKG |
| 17:30 | Reported Tables and PM Checksums (stack SPIRE_Report_tables_v4.ws042) Reported tables are all as expected. |  |  | SDS/TWG/DKG |
| 17:32 | checking the memory (after reporting the latest SPIRE tables): CHECKSUMS all nominal Now recheck the INIT segment of PM by sending from manual command stack TC(6,9) CHECK_MEMORY: SCM02500 SPM9N500 = 0x0 SPMAN500 = 0x4000 SPMBN500= 0x1795 The memory checkword is the penultimate 16-bit word in the received TM(6,10) packet, verify it contains the value: 0x4C5F Check the PCMO segment of PM by sending from manual command stack TC(6,9) CHECK_MEMORY: SCM02500 SPM9N500 = 0x0 SPMAN500 = 0x6000 SPMBN500= 0xFFFF The memory checkword is the penultimate 16-bit word in the received TM(6,10) packet, verify it contains the value: 0x078F Check the PCMO segment of PM by sending from manual command stack TC(6,9) CHECK_MEMORY: SCM02500 SPM9N500 = 0x0 SPMAN500 = 0x15FFF SPMBN500= 0xE14 The memory checkword is the penultimate 16-bit word in the received TM(6,10) packet, verify it contains the value: 0xD362 |  |  | SDS/TWG/DKG |
| 17:45 | H_COP_SPI_MPHN (SPIRE-CP-PDET-ON) OBSID: 0x00002098 The photometer detectors are on. Phases: PSW: 182.12 PMW: 182.12 PLW: 186.35 The optimum phase were set to the values recommended by Jamie Bock PSWJFET1V: -1.59 PSWJFET2V: -1.37 PSWJFET3V: -1.18 PSWJFET4V: -1.47 PSWJFET5V: -1.68 PSWJFET6V: -1.59 PMWJFET1V: -1.68 PMWJFET2V: -1.88 PMWJFET3V: -1.78 PMWJFET4V: -1.59 PLWJFET1V: -1.78 PLWJFET2V: -1.37 TCJFETV: -1.08 |  |  | SDS/TWG/DKG |
| 17:57 | PTC Test 1: OBSID: 0x00002099 TPF version 6 - Started VM at 18:08. - End 18:46 HCSPTCTE_50976753_0006.IPF 2009-07-01T13:23:00Z 1 R FFFFFF6A H Delta required temperature (ADC units) 2 R A8F00000 H PTC SUBKTEMP get temperature command 3 R 200000 D PTC Loop period (us) 4 R 40F00000 H Kp (PID parameter)- float 5 R 3E4CCCCD H Ki (PID parameter)- float 6 R 3B23D70A H Kd (PID parameter)- float 7 R 47435000 H Ki limit - float 8 R 3FC8547F H Low pass filter gain - float 9 R BF924D2F H Low pass filter coefficient b1 - float 10 R BED35ABB H Low pass filter coefficient b2 - float 11 R 0 D DAC constant offset - float 12 R 2048 D Max DAC value - int 13 R 1 D Pulse Width Modulation (PWM) flag 14 R 5 D TM flag (# VM sci frames/VM TM Pkt) 15 R 100 D Init count - try > 2 16 R 0 D Error Compensation Parameter – float This gave very good performance, with SUBKTEMP stabilisation in only a few minutes. Other indicators stabilised after about 10 minutes. TPF version 7 - Started VM at 18:47. - End 19:30 HCSPTCTE_50976753_0007.IPF 2009-07-01T13:50:00Z 1 R 00000032 H Delta required temperature (ADC units) 2 R 8C3F001A H PTC get temperature command 3 R 200000 D PTC Loop period (us) 4 R BD800000 H Kp (PID parameter)- float 5 R BD4CCCCD H Ki (PID parameter)- float 6 R BB23D70A H Kd (PID parameter)- float 7 R 47435000 H Ki limit - float 8 R 3FC8547F H Low pass filter gain - float 9 R BF924D2F H Low pass filter coefficient b1 - float 10 R BED35ABB H Low pass filter coefficient b2 - float 11 R 0 D DAC constant offset - float 12 R 2048 D Max DAC value - int 13 R 1 D Pulse Width Modulation (PWM) flag 14 R 5 D TM flag (# VM sci frames/VM TM Pkt) 15 R 100 D Init count - try > 2 16 R 0 D Error Compensation Parameter - float TPF version 8 - A higher delta T case to simulate operations in the first few hours after a cooler recycle. - Started VM at 19:32 HCSPTCTE_50976753_0008.IPF 2009-07-01T13:57:00Z 1 R FFFFFC18 H Delta required temperature (ADC units) 2 R A8F00000 H PTC SUBKTEMP get temperature command 3 R 200000 D PTC Loop period (us) 4 R 40F00000 H Kp (PID parameter)- float 5 R 3E4CCCCD H Ki (PID parameter)- float 6 R 3B23D70A H Kd (PID parameter)- float 7 R 47435000 H Ki limit - float 8 R 3FC8547F H Low pass filter gain - float 9 R BF924D2F H Low pass filter coefficient b1 - float 10 R BED35ABB H Low pass filter coefficient b2 - float 11 R 0 D DAC constant offset - float 12 R 2048 D Max DAC value - int 13 R 1 D Pulse Width Modulation (PWM) flag 14 R 5 D TM flag (# VM sci frames/VM TM Pkt) 15 R 100 D Init count - try > 2 16 R 0 D Error Compensation Parameter - float Noted that no science packets were sent to ground on VC1 for the first 10 minutes of this test. MOC forgot to remove the MTL commands to disable the VC1 at the end of the original DTCP (at 19:30z), and hence the real time science data was lost without any action from the ground. Ended the test sequence in OBSID 0x00002099 Started a new OBSID for further tests once the heater has cooled down. This is due to the high delta T case in TPF v8. OBSID: 0x0000209A Delay added as SOE requested a 5 minute break. TPF version 9 - Started VM at 20:21 HCSPTCTE_50976753_0009.IPF 2009-07-01T14:01:00Z 1 R FFFFFF6A H Delta required temperature (ADC units) 2 R A8F00000 H PTC get temperature command 3 R 200000 D PTC Loop period (us) 4 R 3FF00000 H Kp (PID parameter)- float 5 R 3D4CCCCD H Ki (PID parameter)- float 6 R 3B23D70A H Kd (PID parameter)- float 7 R 47435000 H Ki limit - float 8 R 3FC8547F H Low pass filter gain - float 9 R BF924D2F H Low pass filter coefficient b1 - float 10 R BED35ABB H Low pass filter coefficient b2 - float 11 R 0 D DAC constant offset - float 12 R 2048 D Max DAC value - int 13 R 1 D Pulse Width Modulation (PWM) flag 14 R 5 D TM flag (# VM sci frames/VM TM Pkt) 15 R 100 D Init count - try > 2 16 R 0 D Error Compensation Parameter - floa Noted that OBSID is still showing as 0x00002099, so it’s not clear that a new one was set. |  |  | TWG/DKG |
| 20:35 | By emergency request, the test was aborted at 20:35. The ground station reported loss of coverage expected in 5 minutes time, and so the spacecraft had to be put into a safe state before LOS. The procedure HSCPTCTE was finished, and TF and TG were executed stop switch off the PTC heater and stop the detector data. REDB was then executed to put the HK back to nominal 0.25Hz HK rate. There was no time for detector or BSM switch-off. |  |  | TWG/DKG |
| 21:00 | A new ground station was obtained by MOC in order to finish the tests. A sequence was constructed to put SPIRE back into PHOT_STBY, and then to resume the final PTC test case: - SETB - Set_OBS_Mode (0x300) - HSCPTCT - HSCPTCTC - HSCPTCTD - HSCPTCTE Instrument telemetry resumed and all appears nominal. The BSM is still in closed loop, and the photometer detectors are switched on and at nominal settings. The above sequence was executed successfully. Test OBSID: 0x0000209D TPF version 9 - Repeat of the aborted test case on SUBKTEMP - Started VM at 21:30 MOC informed us that LOS would be at 22:14, so the procedure would need to end before 22:00. - End 21:55 |  |  | TWG/DKG |
| 21:57 | Started the switch-off process and transition back to PHOT_STBY. OBSID: 0x0000209E (PDET_OFF) OBSID: 0x0000209F (BSM_OFF) SPIRE back to REDY mode. Tests finished for today. |  |  | TWG/DKG |